Poznamky z chapter 6

Router(config)# ip route network-address subnet-mask { ip-address | interface-type interface-number [ ip-address ]} [ distance ] [ name name ] [ permanent ] [ tag tag ]

Distance = admin. vzdialenost/

nedoveryhodnost

Along with ping and traceroute, useful commands to verify static routes include:

show ip route

show ip route static

show ip route network

DEFAULT ROUTE

A default route is a static route that matches all packets. A default route is used when no other routes in the routing table match the destination IP address of the packet.

By default, static routes have an administrative distance of 1

Konfiguracia predvolenej (default) a záložnej(plávajúcej-floating) cesty

What would happen if R2 failed? To simulate this failure both serial interfaces of R2 are shut down

Common IOS troubleshooting commands include:

ping

traceroute

show ip route

show ip interface brief

show cdp neighbors detail

The incorrect route is removed and the correct route is then entered.